MC Practicles

Exp1 CDMA

Code:-

import numpy as np

c1=[1,1,1,1]

c2=[1,-1,1,-1]

c3=[1,1,-1,-1]

c4=[1,-1,-1,1]

rc=[]

print("Enter the data bits :")

d1=int(input("Enter D1 :"))

d2=int(input("Enter D2 :"))

d3=int(input("Enter D3 :"))

d4=int(input("Enter D4 :"))

r1=np.multiply(c1,d1)

r2=np.multiply(c2,d2)

r3=np.multiply(c3,d3)

r4=np.multiply(c4,d4)

result_channel= r1+r2+r3+r4;

print("Resultant Channel", result_channel)

Channel=int(input("Enter the station to listen for c1=1,c2=2,c3=3,c4=4 :"))

if Channel==1:

rc=c1

elif Channel==2:

rc=c2

elif Channel==3:

rc=c3

elif Channel==4:

rc=c4

inner_product=np.multiply(result_channel,rc)

print("Inner Product", inner_product)

res1=sum(inner_product)

data=res1/len(inner_product)

print("Data bit that was sent", data)

EXP 3 BER

Code:-

clear all;

close all;

clc;

% Simulation Parameters

M = 2; % BPSK

Eb_N0_dB = 0:60; % SNR in dB

Eb_N0 = 10.^(Eb_N0_dB/10); % SNR

N = 10000; % number of bits

% BPSK Modulation

x = rand(1,N)>0.5; % random bit stream

y = 2*x - 1; % BPSK modulation

% Rayleigh Fading Channel

h = (randn(1,N) + 1i*randn(1,N))/sqrt(2); % Rayleigh fading channel

% Initialize BER

BER = zeros(1,length(Eb_N0_dB));

% Simulation

for ii = 1:length(Eb_N0_dB)

% Add noise

n = (randn(1,N) + 1i*randn(1,N))/sqrt(2)*sqrt(1/(2*Eb_N0(ii)));

r = h.*y + n; % received signal

% Demodulation

r_det = real(r) > 0; % decision detector

% Calculate BER

BER(ii) = sum(xor(x,r_det))/N;

end

% Plot BER vs SNR

semilogy(Eb_N0_dB,BER,'-o');

grid on;

xlabel('SNR (dB)');

ylabel('BER');

title('BER vs SNR for BPSK over Rayleigh Fading Channel');

Cisco packet configuration

Exp 7 File Transfer

Server.py

Code:-

import socket

HOST=socket.gethostbyname(socket.gethostname())

PORT=4455

with socket.socket(socket.AF_INET,socket.SOCK_STREAM) as s:

s.bind((HOST,PORT))

s.listen()

print("server listing on",HOST,"port",PORT)

conn,addr=s.accept()

print("connected by ",addr)

with open(“file_to_send”,"rb") as f:       #file location

while True:

data=f.read(1024)

if not data:

break

conn.sendall(data)

print("File send successfully")

Client.py

Code:-

import socket

#HOST='192.168.2.104'

HOST=socket.gethostbyname(socket.gethostname())

PORT=4455

with socket.socket(socket.AF_INET,socket.SOCK_STREAM) as s:

s.connect((HOST,PORT))

with open("received_file","wb") as f:                #file location

while True:

data=s.recv(1024)

if not data:

break

f.write(data)

print("File Received successfully")